Disciplinary Case Management & Monitoring System (DCMMS)

Ministry of Education, Sri Lanka

Complete Server Building & Hosting Technical Guide

| System Name: | Disciplinary Case Management & Monitoring System (DCMMS) |
| --- | --- |
| Target Load: | Minimum 100 Concurrent Active Users (Node.js + PostgreSQL Stack) |

# 1. Recommended Hardware Specs by Use Case

Selected specifications based on operational load profiles:

| Component | Small Web Server (Light App) | DCMMS Server (100+ Concurrent Users) | High Performance / Virtualization |
| --- | --- | --- | --- |
| Primary Use | Dev & Staging (<10 users) | Production DCMMS Deployment (100+ Users) | Multi-Dept Virtualization (Proxmox) |
| CPU | 2 to 4 Cores (i3 / N100 / Ryzen 3) | 6 to 8 Cores / 12-16 Threads (Intel i7 13th Gen / Ryzen 7 7700) | 8 to 16+ Cores (Ryzen 9 / i9 / Xeon) |
| RAM | 8 GB - 16 GB (DDR4/DDR5) | 16 GB - 32 GB (DDR4/DDR5) | 64 GB - 128 GB+ (ECC Recommended) |
| Boot Drive (OS & DB) | 256 GB - 512 GB NVMe SSD | 1 TB PCIe 4.0 NVMe M.2 SSD (High IOPS) | 1 TB - 2 TB NVMe SSD (PCIe Gen4 RAID 1) |
| Bulk Storage / Backups | 1 TB SATA SSD / HDD | 2 x 4 TB NAS HDDs (RAID 1 Mirror) | Multi-bay HDD Array (ZFS RAIDZ2) |
| Network | 1 Gbps Ethernet | 1 Gbps Ethernet (100 Mbps Fiber Upload) | 2.5 Gbps / 10 Gbps Ethernet |
| Power Supply (PSU) | 300W - 450W 80+ Bronze | 500W - 650W 80+ Gold | 750W+ 80+ Platinum / Redundant |

# 2. Component Breakdown Explained

## A. Operating System (OS) Recommendations

Ubuntu Server 24.04 LTS (Recommended for DCMMS): Best for running Docker containers, Node.js runtime, Next.js, and PostgreSQL. Massive community support, long-term support (LTS), light resource overhead (~500MB RAM headless).

Debian 12 (Bookworm): Best for rock-solid industrial stability and minimalistic headless setups.

Proxmox VE (Hypervisor): Best for running multiple isolated Virtual Machines (VMs) and LXC containers on one physical server.

TrueNAS SCALE: Best for dedicated storage management, NAS setup, and ZFS data redundancy.

## B. RAM (Random Access Memory)

Capacity vs Speed: Capacity is far more critical than raw speed for server stability. Running out of RAM causes the Linux OOM killer to terminate database processes.

100 User Sizing: 32 GB RAM allows allocating 8 GB to PostgreSQL, 4 GB to Next.js process cluster, 2 GB to Redis caching, and keeps 18 GB headroom for buffer caches and system peak spikes.

ECC RAM: Error-Correcting Code RAM prevents silent memory data corruption. Recommended for government database operations.

## C. Storage (Boot Drive & Bulk Storage)

Boot Drive (OS & PostgreSQL DB): Always use a PCIe 4.0 NVMe M.2 SSD (e.g., Samsung 990 Pro / WD Black SN850X). Concurrent database transactions generated by 100 users require high random read/write IOPS (>500k IOPS).

Bulk Data Drive: Use dedicated NAS Grade Hard Drives (such as Western Digital Red Plus or Seagate IronWolf) in RAID 1 for daily database backups, system logs, and uploaded PDF case attachments.

## D. Network & Power Protection

Ethernet Connection: Connect the server via Cat6 / Cat6a Gigabit Ethernet cable. Never use Wi-Fi for server infrastructure.

Uninterruptible Power Supply (UPS): A critical addition (e.g., APC / CyberPower 600VA-1500VA). Protects server hardware against sudden grid power failure and executes safe automated shutdowns via USB signaling (apcupsd).

# 3. Operating System & Hypervisor Matrix

| OS / Platform | Best Used For | Key Strengths | Considerations |
| --- | --- | --- | --- |
| Ubuntu Server 24.04 LTS | DCMMS Hosting, Docker, PostgreSQL | Massive community, extensive docs, low memory footprint (~500MB) | Frequent package updates require planned maintenance |
| Debian 12 (Bookworm) | Ultra-stable Production Servers | Industrial-grade stability, minimal resource usage | Older package versions in default repos |
| Proxmox VE (PVE) | Virtualization & Multi-OS Infrastructure | Web management GUI, LXC containers, KVM VMs, built-in backups | Higher initial administration learning curve |
| TrueNAS SCALE | Dedicated File & Backup Storage | Native ZFS support, SMB/NFS file shares, storage GUI | Requires dedicated RAM (16GB minimum for ZFS) |

Official Recommendation: For the DCMMS web application stack (Next.js 16 + Prisma + PostgreSQL), Ubuntu Server 24.04 LTS paired with Docker & Docker Compose offers the highest stability, deployment velocity, and performance for 100+ concurrent users.

# 4. Physical Infrastructure & Environment

Wired Connectivity: Always connect the server using a Cat6 / Cat6a Ethernet cable. Avoid Wi-Fi due to latency, interference, and packet loss.

Cooling & Thermals: Maintain ambient room temperature below 28°C (82°F). Equip the server chassis with static pressure cooling fans (e.g., Noctua or Arctic) for 24/7 continuous operation.

Headless Management: Disconnect monitor, keyboard, and mouse after initial OS installation. Administrate the server remotely via SSH or web management consoles.

# 5. Network Setup & Public Access Architecture

Static IP Assignment: Configure a static internal IP address (e.g., 192.168.1.100) via MAC address reservation on the Ministry router.

Reverse Proxy: Deploy Nginx Proxy Manager or Caddy to handle SSL certificates (HTTPS) automatically via Let's Encrypt and route inbound web requests to internal container ports.

Cloudflare Tunnels: Safely exposes the DCMMS web interface to the public/ministry network without opening inbound ports on the router or exposing the origin server IP address.

Tailscale / WireGuard VPN: Restricted administrative access for IT personnel to manage the server and SSH ports remotely.

# 6. Security Hardening Checklist

Disable Root SSH Access: Edit /etc/ssh/sshd_config and set PermitRootLogin no.

Enforce SSH Key Authentication: Generate keypairs locally (ssh-keygen), copy public key (ssh-copy-id), and set PasswordAuthentication no.

Configure Uncomplicated Firewall (UFW): Block all incoming traffic by default, allowing only port 22 (SSH), 80 (HTTP), and 443 (HTTPS).

Install Fail2ban: Protect against brute-force password attacks by automatically banning offending IP addresses.

Automated Security Patches: Enable unattended-upgrades to automatically install Linux kernel security patches.

# 7. Software Architecture & Containerization Stack

Using Docker & Docker Compose isolates application services, preventing system file corruption and simplifying system upgrades.

## Recommended DCMMS Docker Stack Components:

nginx-proxy-manager: Reverse proxy & automated SSL certificate renewal.

dcmms-nextjs-app: Next.js 16 application runtime container (scaled to 3 active worker replicas).

redis: In-memory cache layer for user session management and fast lookup queries.

pgbouncer: Connection pooler to multiplex 100+ concurrent DB connections cleanly.

postgresql: Primary relational database storing disciplinary case records.

portainer: Web-based container management GUI.

watchtower: Automated container security maintenance.

# 8. Monitoring, Alerting & 3-2-1 Backup Strategy

## Real-Time Monitoring

Uptime Kuma: Continuously monitors HTTP endpoint health and sends immediate alerts (via Email/Telegram) if the application becomes unreachable.

Netdata: Light, zero-overhead real-time dashboard tracking CPU usage, memory allocation, storage IOPS, and server temperatures.

## 3-2-1 Data Backup Strategy for DCMMS

3 Copies of Data: Primary production database + Local backup + Offsite cloud backup.

2 Different Media Types: High-speed NVMe system drive + Local NAS Hard Drive array.

1 Offsite Encrypted Backup: Automated daily encrypted backups (using Restic or BorgBackup) synced to offsite cloud storage (e.g., Backblaze B2 or S3 compatible storage).